UNIVERSITY CAMPUS NETWORK USING CISCO PACKET TRACER

Network Design, Configuration, Simulation and Testing — Final Report

| Field | Details |
| --- | --- |
| Simulation Platform | Cisco Packet Tracer 9.0.0 |
| Project File | University_Campus_Network_Premier_University.pkt |

Contents

(Right-click the table and choose Update Field in Microsoft Word to refresh page numbers.)

# 1. Abstract

This report documents the design, configuration and testing of a simulated smart university campus network for Premier University, Chittagong, built directly in Cisco Packet Tracer 9.0. The network contains 31 devices: a simulated ISP and Internet server, a Cisco 2911 edge router, two Catalyst 3650 multilayer distribution switches, five Catalyst 2960 access switches, four data-centre servers, seven PCs, a WPA2 access point with two wireless clients, and four IoT devices. Nine VLANs are addressed from 10.10.0.0/16 with VLSM, routed with OSPFv2 (IPv4) and OSPFv3 (IPv6), served by a central DHCP/DNS server through DHCP relay, protected by three named extended ACLs, an SSH-only management policy and NAT/PAT, and extended with an IoT registration server that switches classroom lighting by occupancy. Of 31 planned tests, 23 passed, 2 failed and 6 were not run; every result is reported as observed, with screenshots captured from the running simulation.

# 2. Introduction

University campuses combine user groups with very different needs — students, registrar and administrative staff, faculty, researchers, facilities and IT — on one shared infrastructure. The project brief for CSE 3568 asks for a comprehensive network that demonstrates multi-zone routing, VLAN segmentation, VLSM and IPv6 addressing, wireless access, ACL and NAT security, and an IoT automation function. The smart-city scenario in the course template was adapted to a smart campus, as described in the approved-for-review proposal. All campus segments are simulated; they are not a description of the university's real infrastructure.

# 3. Problem Statement

Design and build, in Cisco Packet Tracer, a campus network in which: (i) every authorised segment can reach the services it needs; (ii) students cannot reach administrative, management or IoT segments; (iii) clients are addressed automatically from a central server; (iv) the campus reaches a simulated Internet through a single translated public address while only the web portal is published inward; (v) at least one segment runs IPv6 end to end; (vi) students can connect over secured Wi-Fi; and (vii) a smart-campus automation rule reacts to a sensor condition.

# 4. Objectives

- Build a routed two-block campus with OSPF adjacencies reaching FULL state between all Layer-3 devices.

- Implement a non-overlapping VLSM plan for nine VLANs and three /30 links, plus dual-stack IPv6 on the library and server VLANs.

- Enforce an inter-zone security policy with named ACLs, an SSH-only management ACL and NAT/PAT, verified by permit and deny tests.

- Provide WPA2-AES student Wi-Fi and centralised DHCP/DNS through DHCP relay.

- Demonstrate occupancy-based lighting and temperature-based ventilation rules on a central IoT server.

# 5. Scope and Assumptions

- All segments, addresses and services are simulated; public addresses use RFC 5737 (203.0.113.0/29, 198.51.100.0/24) and IPv6 uses the RFC 3849 documentation prefix 2001:DB8::/32.

- The two building blocks are joined by routed links in one OSPF area; there are no WAN serial links between campuses.

- IPv6 is internal only (VLAN 50, VLAN 60 and link L3); no IPv6 Internet service is simulated.

- Lab-only credentials (netadmin / PULab2026, Wi-Fi PUStudent2026, IoT iotadmin / PUIoT2026) are used for demonstration and must be changed before any reuse.

- Packet Tracer feature support defines the limits of the design; differences from the proposal are listed in Section 22.

# 6. Requirements

| ID | Requirement | Status |
| --- | --- | --- |
| FR1 | Inter-zone routing with OSPF and a redundant inter-block path | Implemented; adjacencies verified (failover not tested) |
| FR2 | VLAN segmentation, 802.1Q trunks with native VLAN 999, SVI inter-VLAN routing | Implemented and verified by DHCP/ping tests |
| FR3 | Central DHCP (relay) and DNS | Implemented and verified |
| FR4 | WPA2-AES student wireless in VLAN 40 | Implemented; association and DHCP verified |
| FR5 | ACLs, SSH-only management, NAT/PAT and static NAT | Implemented and verified (some negative tests not run) |
| FR6 | Dual-stack IPv6 with OSPFv3; IoT automation | Implemented; IPv6 ping and motion rule verified; temperature rule not triggered |

# 7. Network Architecture

The design is a collapsed core/distribution hierarchy in two blocks. R-EDGE (Cisco 2911) is the only Internet boundary and performs NAT/PAT and inbound filtering. Two Catalyst 3650-24PS multilayer switches act as the distribution and inter-VLAN routing layer for the Academic & Library block (DSW-ACAD) and the Administration & Data Center block (DSW-ADMIN). The three Layer-3 devices form a triangle of routed /30 links (L1, L2, L3) in OSPF Area 0, so traffic between blocks normally uses L3 and can fall back through R-EDGE. VLANs are local to each block, which keeps every spanning-tree domain small. Access is provided by five Catalyst 2960-24TT switches.

Figure 1. As-built logical design (diagram drawn from the implemented configuration).

# 8. Actual Packet Tracer Topology

Figure 2 is a screenshot of the logical workspace of the saved project file. Device labels match the configured hostnames; port labels show the cabling used.

Figure 2. Logical topology in Cisco Packet Tracer 9.0 (screenshot of the saved project).

# 9. Device Inventory

| Device | Model | Role | Key interfaces / address |
| --- | --- | --- | --- |
| ISP-R | Cisco 2911 | Simulated ISP | G0/0 203.0.113.1/29, G0/1 198.51.100.1/24 |
| INTERNET-SRV | Server-PT | Simulated Internet web server | 198.51.100.10/24 |
| R-EDGE | Cisco 2911 | Edge router, NAT/PAT, EDGE-IN ACL, OSPF ASBR | G0/0 203.0.113.2/29, G0/1 10.10.255.1/30, G0/2 10.10.255.5/30, Lo0 10.10.254.1 |
| DSW-ACAD | Catalyst 3650-24PS | Distribution, SVIs 30/40/50/70/98 | G1/0/1 10.10.255.2, G1/0/2 10.10.255.9, trunks G1/0/3-5 |
| DSW-ADMIN | Catalyst 3650-24PS | Distribution, SVIs 10/20/60/99 | G1/0/1 10.10.255.6, G1/0/2 10.10.255.10, trunks G1/0/3-4 |
| ASW-LAB / ASW-LIB / ASW-IOT | Catalyst 2960-24TT | Access (academic block) | Fa0/1 trunk; mgmt 10.10.4.66/.67/.68 |
| ASW-ADMIN / ASW-DC | Catalyst 2960-24TT | Access (admin block) | Fa0/1 trunk; mgmt 10.10.4.82/.83 |
| SRV-DHCP-DNS / SRV-WEB / SRV-IOT / SRV-NMS | Server-PT | DHCP+DNS / portal / IoT controller / syslog | 10.10.4.34 / .35 / .36 / .37 |
| AP-STUDENT | AccessPoint-PT | WPA2-AES SSID PU-STUDENT | ASW-LIB Fa0/3, VLAN 40 |
| PC-LAB1, PC-LAB2, PC-LIB1, PC-ADMIN1, PC-FAC1, NMS-PC | PC-PT | Representative users | DHCP (NMS-PC static 10.10.4.90) |
| STU-TABLET1, STU-PHONE1 | TabletPC-PT, Smartphone-PT | Wireless clients | DHCP 10.10.0.11, 10.10.0.13 |
| IOT-MOTION-1, IOT-LIGHT-1, IOT-TEMP-1, IOT-FAN-1 | Home IoT devices | Sensors and actuators | Wired adapter PT-IOT-NM-1CFE, VLAN 70 |

# 10. VLAN Table

| VLAN | Name | Purpose | Switches | Gateway (SVI) |
| --- | --- | --- | --- | --- |
| 10 | ADMIN | Administration & Registrar | DSW-ADMIN, ASW-ADMIN | 10.10.3.193 |
| 20 | FACULTY | Faculty & staff | DSW-ADMIN, ASW-ADMIN | 10.10.3.1 |
| 30 | ACAD-LAB | Computer labs | DSW-ACAD, ASW-LAB | 10.10.2.1 |
| 40 | STU-WIFI | Student wireless | DSW-ACAD, ASW-LIB | 10.10.0.1 |
| 50 | LIBRARY | Library & research (dual-stack) | DSW-ACAD, ASW-LIB | 10.10.3.129 |
| 60 | SERVERS | Data centre (dual-stack) | DSW-ADMIN, ASW-DC | 10.10.4.33 |
| 70 | IOT | Smart campus devices | DSW-ACAD, ASW-IOT | 10.10.4.1 |
| 98 | MGMT-ACAD | Switch management (academic) | DSW-ACAD + academic ASWs | 10.10.4.65 |
| 99 | MGMT-ADMIN | Switch management + NMS-PC | DSW-ADMIN + admin ASWs | 10.10.4.81 |
| 998 | PARKING | Unused, shut-down ports | All switches | — |
| 999 | NATIVE | Unused native VLAN on trunks | All trunks | — |

# 11. IPv4 and VLSM Addressing Table

Subnets were allocated largest first from 10.10.0.0/16. Every subnet was checked by script for overlap and DHCP-scope containment before configuration. VLANs 98 and 99 occupy one aligned /27 (10.10.4.64/27), so one ACL wildcard (0.0.0.31) matches both management VLANs. 10.10.4.96 – 10.10.253.255 remains free.

| VLAN/Link | Name | Network | Prefix | Subnet mask | First usable | Last usable | Broadcast | DHCP range |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | ADMIN (Administration & Registrar) | 10.10.3.192 | /26 | 255.255.255.192 | 10.10.3.193 | 10.10.3.254 | 10.10.3.255 | 10.10.3.201-10.10.3.254 |
| 20 | FACULTY (Faculty & Staff) | 10.10.3.0 | /25 | 255.255.255.128 | 10.10.3.1 | 10.10.3.126 | 10.10.3.127 | 10.10.3.11-10.10.3.126 |
| 30 | ACAD-LAB (Academic Bldg & Computer Labs) | 10.10.2.0 | /24 | 255.255.255.0 | 10.10.2.1 | 10.10.2.254 | 10.10.2.255 | 10.10.2.11-10.10.2.254 |
| 40 | STU-WIFI (Student Wi-Fi) | 10.10.0.0 | /23 | 255.255.254.0 | 10.10.0.1 | 10.10.1.254 | 10.10.1.255 | 10.10.0.11-10.10.1.254 |
| 50 | LIBRARY (Library & Research) | 10.10.3.128 | /26 | 255.255.255.192 | 10.10.3.129 | 10.10.3.190 | 10.10.3.191 | 10.10.3.136-10.10.3.190 |
| 60 | SERVERS (Server Room / Data Center) | 10.10.4.32 | /28 | 255.255.255.240 | 10.10.4.33 | 10.10.4.46 | 10.10.4.47 | static |
| 70 | IOT (Smart Campus IoT) | 10.10.4.0 | /27 | 255.255.255.224 | 10.10.4.1 | 10.10.4.30 | 10.10.4.31 | 10.10.4.11-10.10.4.30 |
| 98 | MGMT-ACAD (Network Mgmt (Academic block)) | 10.10.4.64 | /28 | 255.255.255.240 | 10.10.4.65 | 10.10.4.78 | 10.10.4.79 | static |
| 99 | MGMT-ADMIN (Network Mgmt & Security) | 10.10.4.80 | /28 | 255.255.255.240 | 10.10.4.81 | 10.10.4.94 | 10.10.4.95 | static |
| L1 | R-EDGE G0/1 ↔ DSW-ACAD G1/0/1 | 10.10.255.0 | /30 | 255.255.255.252 | 10.10.255.1 | 10.10.255.2 | 10.10.255.3 | - |
| L2 | R-EDGE G0/2 ↔ DSW-ADMIN G1/0/1 | 10.10.255.4 | /30 | 255.255.255.252 | 10.10.255.5 | 10.10.255.6 | 10.10.255.7 | - |
| L3 | DSW-ACAD G1/0/2 ↔ DSW-ADMIN G1/0/2 | 10.10.255.8 | /30 | 255.255.255.252 | 10.10.255.9 | 10.10.255.10 | 10.10.255.11 | - |
| Lo0 | OSPF router-IDs | 10.10.254.1-3 | /32 | 255.255.255.255 |  |  |  | - |
| WAN | R-EDGE G0/0 - ISP-R G0/0 | 203.0.113.0 | /29 | 255.255.255.248 | 203.0.113.1 | 203.0.113.6 | 203.0.113.7 | - |
| ISP LAN | Simulated Internet | 198.51.100.0 | /24 | 255.255.255.0 | 198.51.100.1 | 198.51.100.254 | 198.51.100.255 | - |
| Reserved | Future DMZ / server growth | 10.10.4.48 | /28 | 255.255.255.240 |  |  |  |  |

# 12. IPv6 Configuration

| Segment | Prefix | Router address | Host |
| --- | --- | --- | --- |
| VLAN 50 Library | 2001:DB8:ACAD:50::/64 | DSW-ACAD SVI ::1, link-local FE80::1 | PC-LIB1 2001:DB8:ACAD:50::10 (static), gateway FE80::1 |
| VLAN 60 Servers | 2001:DB8:ACAD:60::/64 | DSW-ADMIN SVI ::1, link-local FE80::1 | SRV-WEB 2001:DB8:ACAD:60::35, gateway FE80::1 |
| Link L3 | 2001:DB8:ACAD:FF00::/64 | DSW-ACAD ::1, DSW-ADMIN ::2 | — |

ipv6 unicast-routing and OSPFv3 process 10 (Area 0) run on both 3650 switches. The OSPFv3 adjacency reached FULL and DSW-ACAD learned 2001:DB8:ACAD:60::/64 (Figure 9a). PC-LIB1 did not obtain a global address by SLAAC, so a static address was configured; the IPv6 ping to the web server then succeeded (Figure 15).

Figure 15. PC-LIB1: static IPv6 address and 0% loss ping to SRV-WEB 2001:DB8:ACAD:60::35.

# 13. Routing Design

OSPFv2 process 10 runs in a single Area 0 with loopback router-IDs 10.10.254.1–3. On the 3650s, passive-interface default is used and only the two routed uplinks are made active, so SVI subnets are advertised without forming adjacencies toward users. R-EDGE holds a static default route to the ISP and advertises it with default-information originate. OSPF was chosen over EIGRP (vendor-specific) and RIP (hop-count metric, slow convergence). The ISP router intentionally has no route to 10.10.0.0/16, so Internet replies can return only through NAT.

Figure 3. DSW-ADMIN: OSPFv2 neighbours (R-EDGE, DSW-ACAD) and OSPFv3 neighbour FULL; block VLANs.

Figure 4. R-EDGE routing table: every campus subnet learned by OSPF, and the static default route.

# 14. DHCP and DNS

SRV-DHCP-DNS (10.10.4.34) holds one pool per client VLAN (10, 20, 30, 40, 50, 70). Each SVI forwards DHCP broadcasts with ip helper-address 10.10.4.34, and the server selects the pool that matches the relay address. DNS A records point portal.pu.internal, iot.pu.internal and nms.pu.internal at the servers, and an AAAA record (portal6.pu.internal) holds the portal's IPv6 address. Observed leases: PC-LAB1 10.10.2.11, PC-LAB2 10.10.2.12, PC-ADMIN1 10.10.3.201, PC-FAC1 10.10.3.11, PC-LIB1 10.10.3.136, tablet 10.10.0.12 (10.10.0.11 after re-lease), phone 10.10.0.13, motion detector 10.10.4.11.

Figure 5a. DHCP pools on SRV-DHCP-DNS.

Figure 5b. DNS resource records.

Figure 6. PC-LAB1 lease from the central server (10.10.4.34) through DHCP relay.

# 15. Server Configuration

SRV-WEB serves a campus portal page over HTTP on 10.10.4.35 and 2001:DB8:ACAD:60::35, and is published to the outside as 203.0.113.5 by static NAT. SRV-IOT runs the IoT registration service. SRV-NMS has its syslog service enabled; no network device has yet been configured to send logs to it (logging host is a recommended next step). INTERNET-SRV (198.51.100.10) represents a public website behind ISP-R.

Figure 7. Campus portal opened by name from PC-LAB1 (DNS + inter-VLAN routing + HTTP).

# 16. ACL and NAT/PAT

| Policy | Applied | Rules (summary) |
| --- | --- | --- |
| STUDENT-IN (extended) | DSW-ACAD SVI 30 and SVI 40, inbound | Permit DHCP; permit established TCP and echo-replies to faculty; deny faculty, admin (10.10.3.192/26), IoT (10.10.4.0/27), management (10.10.4.64/27); permit everything else |
| IOT-PROTECT (extended) | DSW-ACAD SVI 70, inbound | Permit DHCP; permit IoT → SRV-IOT 10.10.4.36; permit echo-replies to management; deny all else |
| EDGE-IN (extended) | R-EDGE G0/0, inbound | Permit HTTP to 203.0.113.5; permit established TCP, DNS replies, echo-reply, unreachable; deny all else |
| MGMT-ONLY (standard) | access-class on VTY lines of all 8 network devices | Permit 10.10.4.64/27 only; VTY accepts SSH v2 only, local login |
| NAT-USERS + PAT | R-EDGE inside G0/1-2, outside G0/0 | Deny IoT and management subnets; permit 10.10.0.0/16; overload on G0/0 (203.0.113.2) |
| Static NAT | R-EDGE | 10.10.4.35 ↔ 203.0.113.5 |

Figure 8a. R-EDGE (SSH from NMS-PC): OSPF neighbours, NAT statistics and translations.

Figure 8b. R-EDGE: NAT-USERS, EDGE-IN and MGMT-ONLY hit counters.

Figure 9a. DSW-ACAD: OSPFv3 neighbour, IPv6 OSPF route and OSPFv2 neighbours.

Figure 9b. DSW-ACAD: STUDENT-IN and IOT-PROTECT hit counters.

Figure 10a. PC-LAB1: gateway, Internet (through PAT) and portal-by-name reachable.

Figure 10b. PC-LAB1: registrar and IoT subnets blocked by STUDENT-IN.

Figure 11a. INTERNET-SRV opens the portal through static NAT (203.0.113.5).

Figure 11b. INTERNET-SRV ping to 203.0.113.5 dropped by EDGE-IN.

# 17. Wireless Network

AP-STUDENT (AccessPoint-PT) is connected to ASW-LIB Fa0/3 in VLAN 40 and broadcasts SSID PU-STUDENT with WPA2-PSK and AES. A tablet and a smartphone were configured with the same SSID and passphrase; both associated and leased addresses from the /23 student pool. STUDENT-IN applies to them exactly as to lab PCs. A tablet was used instead of the proposed laptop because it has a built-in wireless adapter.

Figure 12a. AP-STUDENT security settings.

Figure 12b. STU-TABLET1 associated with WPA2-PSK/AES and leased an address from 10.10.0.0/23.

Figure 12c. STU-TABLET1: portal reachable over Wi-Fi; registrar and SSH to R-EDGE blocked.

# 18. IoT Automation

The four Home IoT devices were switched to the wired PT-IOT-NM-1CFE adapter, cabled to ASW-IOT (VLAN 70), addressed by DHCP and registered with SRV-IOT as remote server. An account was created on the registration server from NMS-PC, and four rules were added: R1-Lights-On / R1-Lights-Off (motion detector controls the light) and R2-Fan-High / R2-Fan-Off (temperature above 30 °C sets the fan to High; below 26 °C turns it off, the gap preventing rapid cycling). Triggering the motion detector (Alt+click) switched the light from Off to On. The temperature rule is configured but was not triggered, because the simulated ambient temperature was not changed.

Figure 13a. All four IoT devices online on the SRV-IOT dashboard (motion detector On).

Figure 13b. Automation rules on SRV-IOT.

Figure 13c. After the motion trigger the smart light is on (topology view).

# 19. Testing Matrix

Thirty-one tests were planned against the functional requirements. Each was run in the simulation unless marked NOT TESTED. Screenshot numbers refer to files in the Screenshots folder.

| ID | Area | Test | Expected | Result |
| --- | --- | --- | --- | --- |
| T01 | Routing | OSPF adjacencies — R-EDGE: show ip ospf neighbor | FULL with 10.10.254.2 and 10.10.254.3 | PASS |
| T02 | Routing | All campus subnets learned — R-EDGE: show ip route | O routes for every VLAN subnet + S* default | PASS |
| T03 | IPv6 | OSPFv3 adjacency and route — DSW-ACAD: show ipv6 ospf neighbor / show ipv6 route ospf | Neighbour FULL; 2001:DB8:ACAD:60::/64 learned | PASS |
| T04 | IPv6 | End-to-end IPv6 — PC-LIB1: ping 2001:DB8:ACAD:60::35 | Replies | PASS |
| T05 | IPv6 | SLAAC on VLAN 50 — PC-LIB1 IPv6 = Automatic | Global address from RA | FAIL |
| T06 | DHCP | Relay to VLAN 30 — PC-LAB1: ipconfig /all | 10.10.2.x/24 from 10.10.4.34 | PASS |
| T07 | DHCP | Relay to VLANs 10/20/50 — PC-ADMIN1, PC-FAC1, PC-LIB1 DHCP | Address in own VLAN | PASS |
| T08 | DHCP | Relay to IoT VLAN 70 through IOT-PROTECT — IOT-MOTION-1 DHCP | 10.10.4.x/27 | PASS |
| T09 | VLAN/Trunk | VLANs and 802.1Q trunks — show vlan brief / show interfaces trunk (DSW-ACAD, DSW-ADMIN, ASW-DC) | Correct VLANs; trunks native 999, pruned lists | PASS |
| T10 | Connectivity | Default gateway — PC-LAB1: ping 10.10.2.1 | Replies | PASS |
| T11 | DNS/Inter-VLAN | Name resolution + server reach — PC-LAB1: ping portal.pu.internal | Resolves to 10.10.4.35, replies | PASS |
| T12 | Web | Campus portal — PC-LAB1 browser http://portal.pu.internal | Portal page | PASS |
| T13 | ACL | Student -> Registrar blocked — PC-LAB1: ping 10.10.3.201 | Fail | PASS |
| T14 | ACL | Student -> IoT blocked — PC-LAB1: ping 10.10.4.11 | Fail | PASS |
| T15 | NAT/PAT | Inside user to Internet — PC-LAB1: ping 198.51.100.10 | Replies (ISP has no route to 10.10.0.0/16) | PASS |
| T16 | Static NAT | Outside access to portal — INTERNET-SRV browser http://203.0.113.5 | Portal page | PASS |
| T17 | ACL | Unsolicited inbound ICMP blocked — INTERNET-SRV: ping 203.0.113.5 | Fail | PASS |
| T18 | Wireless | WPA2-AES association + DHCP — STU-TABLET1 / STU-PHONE1 | Associate, lease 10.10.0.x | PASS |
| T19 | Wireless | Wi-Fi client reaches services; ACL applies — STU-TABLET1: ping portal.pu.internal, ping 10.10.3.201 | Portal replies; registrar blocked | PASS |
| T20 | Wireless | Wrong passphrase rejected — Client with wrong key | No association | NOT TESTED |
| T21 | Management | SSH v2 from management VLAN — NMS-PC: ssh -l netadmin to R-EDGE, DSW-ACAD, DSW-ADMIN, ASW-DC | Login | PASS |
| T22 | Management | SSH from non-management host blocked — STU-TABLET1: ssh -l netadmin 10.10.254.1 | Refused | PASS |
| T23 | Management | Console enable on R-EDGE — R-EDGE CLI tab: enable | Prompt accepts secret | FAIL |
| T24 | IoT | Devices registered — SRV-IOT dashboard from NMS-PC | 4 devices online | PASS |
| T25 | IoT | Motion -> light rule — Alt+click IOT-MOTION-1 | Light turns On | PASS |
| T26 | IoT | Temperature -> fan rule — Raise ambient temperature above 30 C | Fan High | NOT TESTED |
| T27 | Routing | Failover when L3 is shut — shutdown G1/0/2 on DSW-ACAD, traceroute | Traffic via R-EDGE | NOT TESTED |
| T28 | ACL | Faculty-initiated session to lab allowed — PC-FAC1 ping PC-LAB1 | Replies | NOT TESTED |
| T29 | Switching | Port-security / BPDU Guard — Connect extra device / switch | Port err-disabled | NOT TESTED |
| T30 | File | Native .pkt saved — File > Save / Save As | .pkt written by Packet Tracer | PASS |
| T31 | File | Reopen .pkt and re-test — File > Open | Topology and configs persist | NOT TESTED |

# 20. Actual Test Results

| ID | Observed result | Evidence |
| --- | --- | --- |
| T01 | Both FULL; DSW-ACAD and DSW-ADMIN each also show 2 FULL neighbours | 02, 05, 08 |
| T02 | O routes for 10.10.0.0/23 ... 10.10.4.80/28; S* 0.0.0.0/0 via 203.0.113.1 | 04 |
| T03 | 10.10.254.3 FULL on G1/0/2; O 2001:DB8:ACAD:60::/64 [110/2] | 05 |
| T04 | 4/4 replies, 0% loss | 16 |
| T05 | No global address obtained; static 2001:DB8:ACAD:50::10 used instead | 16 |
| T06 | 10.10.2.11 /24, gw 10.10.2.1, DHCP server 10.10.4.34, DNS 10.10.4.34 | 12 |
| T07 | 10.10.3.201 /26, 10.10.3.11 /25, 10.10.3.136 /26 | 10 |
| T08 | 10.10.4.11 /27; IOT-PROTECT bootps entry matches | 07 |
| T09 | All VLANs active; trunks on, 802.1q, native 999, allowed lists as designed | 06, 08, 09 |
| T10 | 3/4 replies (first lost to ARP) | 13 |
| T11 | Resolved; 3/4 replies (first lost to ARP) | 13 |
| T12 | Page displayed | 15 |
| T13 | Destination host unreachable from 10.10.2.1; STUDENT-IN deny 10.10.3.192 matches | 14, 07 |
| T14 | Destination host unreachable; STUDENT-IN deny 10.10.4.0 matches | 14, 07 |
| T15 | 4/4 replies; NAT-USERS permit 8 matches; 4 expired dynamic translations | 13, 02, 03 |
| T16 | Page displayed; translation tcp 203.0.113.5:80 <-> 10.10.4.35:80 listed | 23, 02 |
| T17 | Destination host unreachable from 203.0.113.2; EDGE-IN deny matches | 24, 03 |
| T18 | Tablet 10.10.0.11 /23 (re-leased; earlier .12), phone 10.10.0.13 /23 | 17, 18 |
| T19 | Portal 4/4 replies; registrar blocked by 10.10.0.1 | 19 |
| T20 | Not run | - |
| T21 | Logged in to all four | 02, 05, 08, 09 |
| T22 | % Connection refused by remote host | 19 |
| T23 | Secret rejected 3x when typed through the remote-control session; same secret works over SSH. Cause not determined. | - |
| T24 | IOT-MOTION-1, IOT-LIGHT-1, IOT-TEMP-1, IOT-FAN-1 online | 20 |
| T25 | Light changed from Off to On; dashboard shows motion On | 20, 22 |
| T26 | Rule configured; ambient temperature not changed | 21 |
| T27 | Not run | - |
| T28 | Not run | - |
| T29 | Configured only | - |
| T30 | Binary project file saved to E:\COMPUTER_NETWORK_FINAL PROJECT | - |
| T31 | Not run (file was re-saved after this session's checks) | - |

Summary: 23 PASS, 2 FAIL, 6 NOT TESTED. The first lost ping in T10 and T11 is ARP resolution on first contact, not packet loss on an established path.

# 21. Screenshots and Evidence

All screenshots were captured at full screen resolution from the running Packet Tracer application (CLI font set to 12 pt for legibility) and are stored in the Screenshots folder; they are cropped to the relevant window but otherwise unedited:

- 01_Topology_Logical_View.png

- 02_R-EDGE_OSPF_Neighbors_and_NAT.png

- 03_R-EDGE_Access_Lists_Counters.png

- 04_R-EDGE_Routing_Table.png

- 05_DSW-ACAD_OSPFv2_OSPFv3_Neighbors_IPv6_Route.png

- 06_DSW-ACAD_VLANs_and_Trunks.png

- 07_DSW-ACAD_ACL_Counters.png

- 08_DSW-ADMIN_OSPF_Neighbors_and_VLANs.png

- 09_ASW-DC_VLANs_and_Trunk.png

- 10_SRV-DHCP-DNS_DHCP_Pools.png

- 11_SRV-DHCP-DNS_DNS_Records.png

- 12_PC-LAB1_ipconfig_DHCP_Lease.png

- 13_PC-LAB1_Gateway_Internet_DNS_Ping.png

- 14_PC-LAB1_ACL_Blocks_Registrar_and_IoT.png

- 15_PC-LAB1_Campus_Portal_by_Name.png

- 16_PC-LIB1_IPv6_Ping_to_SRV-WEB.png

- 17_AP-STUDENT_WPA2_AES.png

- 18_STU-TABLET1_Wireless_DHCP.png

- 19_STU-TABLET1_Portal_OK_ACL_and_SSH_Blocked.png

- 20_IoT_Dashboard_Devices_Live_State.png

- 21_IoT_Automation_Rules.png

- 22_IoT_Motion_Light_On_Topology.png

- 23_INTERNET-SRV_Static_NAT_Portal.png

- 24_INTERNET-SRV_Ping_Blocked_by_EDGE-IN.png

Figure 14a. DSW-ACAD: VLANs and 802.1Q trunks (native 999).

Figure 14b. ASW-DC: VLAN port membership and trunk.

# 22. Limitations and Deviations from the Proposal

- Edge router changed from ISR4331 to Cisco 2911: in Packet Tracer 9.0 the ISR4331's third port (G0/0/2) is SFP-only and refused the copper link to DSW-ADMIN.

- Actual ports differ from the proposal: routed links use G0/1, G0/2 on R-EDGE and G1/0/1 (uplink), G1/0/2 (L3) on the 3650s; access-switch uplinks are 2960 Fa0/1 (100 Mbps).

- PC-LIB1 did not receive an IPv6 address by SLAAC; a static IPv6 address was used (T05 FAIL). The cause (e.g. Router Advertisement behaviour of the 3650 SVI in Packet Tracer) was not investigated further.

- The R-EDGE console rejected the enable secret three times when typed through the remote-control session, although the identical secret worked over SSH (T23 FAIL). Verification on R-EDGE was therefore done over SSH from NMS-PC.

- The ICMP keyword time-exceeded is not supported by Packet Tracer 9.0, so traceroute replies from outside are not explicitly permitted by EDGE-IN.

- Not tested: OSPF failover, Wi-Fi wrong-key rejection, faculty-to-lab reachability, port-security/BPDU-guard violations, the temperature/fan rule, and reopening the saved file.

- SRV-NMS syslog/NTP is enabled but no device points at it yet; WPA2-Enterprise was not attempted.

- Device configurations in Device_Configurations are the scripts that were applied, not show running-config exports.

# 23. Conclusion

The project produced a working Packet Tracer model of a segmented, dual-stack smart campus network. OSPF and OSPFv3 converged across a redundant three-node routed core; DHCP relay and DNS served every client VLAN from one server; the security policy blocked students from administrative and IoT segments while keeping shared services and the Internet reachable through PAT; the portal was published safely through static NAT; WPA2 Wi-Fi clients joined the student VLAN; and an IoT rule switched classroom lighting on occupancy. The failed and untested items listed above are the natural next steps, starting with SLAAC on VLAN 50, an OSPF failover test and triggering the ventilation rule.

# 24. References

[1] Cisco Networking Academy, “Cisco Packet Tracer,” Cisco Systems. https://www.netacad.com/cisco-packet-tracer

[2] Cisco Systems, Cisco IOS configuration guides: OSPF, NAT, Access Control Lists, IPv6. https://www.cisco.com

[3] Y. Rekhter et al., “Address Allocation for Private Internets,” RFC 1918, IETF, 1996.

[4] J. Moy, “OSPF Version 2,” RFC 2328, IETF, 1998.

[5] R. Coltun, D. Ferguson, J. Moy and A. Lindem, “OSPF for IPv6,” RFC 5340, IETF, 2008.

[6] G. Huston, A. Lord and P. Smith, “IPv6 Address Prefix Reserved for Documentation,” RFC 3849, IETF, 2004.

[7] J. Arkko, M. Cotton and L. Vegoda, “IPv4 Address Blocks Reserved for Documentation,” RFC 5737, IETF, 2010.

[8] P. Srisuresh and K. Egevang, “Traditional IP Network Address Translator,” RFC 3022, IETF, 2001.

[9] R. Droms, “Dynamic Host Configuration Protocol,” RFC 2131, IETF, 1997.

[10] IEEE Std 802.1Q-2022; IEEE Std 802.11-2020.

# 25. Appendix: Device Configurations

The applied configuration scripts follow. Lab-only credentials are shown because they are part of the simulation; change them before any reuse.

## ISP-R

! ISP-R - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname ISP-R

no ip domain-lookup

banner motd #Simulated ISP router - CSE 3568 campus network project#

interface GigabitEthernet0/0

description WAN link to campus R-EDGE G0/0

ip address 203.0.113.1 255.255.255.248

no shutdown

interface GigabitEthernet0/1

description Simulated Internet server LAN

ip address 198.51.100.1 255.255.255.0

no shutdown

end

write memory

## R-EDGE

! R-EDGE - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname R-EDGE

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

interface Loopback0

description OSPF router-ID

ip address 10.10.254.1 255.255.255.255

interface GigabitEthernet0/0

description WAN to ISP-R G0/0 (NAT outside)

ip address 203.0.113.2 255.255.255.248

ip nat outside

no shutdown

interface GigabitEthernet0/1

description L1 to DSW-ACAD G1/0/1

ip address 10.10.255.1 255.255.255.252

ip nat inside

no shutdown

interface GigabitEthernet0/2

description L2 to DSW-ADMIN G1/0/1

ip address 10.10.255.5 255.255.255.252

ip nat inside

no shutdown

router ospf 10

router-id 10.10.254.1

network 10.10.255.0 0.0.0.3 area 0

network 10.10.255.4 0.0.0.3 area 0

network 10.10.254.1 0.0.0.0 area 0

default-information originate

ip route 0.0.0.0 0.0.0.0 203.0.113.1

ip access-list standard NAT-USERS

deny 10.10.4.0 0.0.0.31

deny 10.10.4.64 0.0.0.31

permit 10.10.0.0 0.0.255.255

ip nat inside source list NAT-USERS interface GigabitEthernet0/0 overload

ip nat inside source static 10.10.4.35 203.0.113.5

ip access-list extended EDGE-IN

permit tcp any host 203.0.113.5 eq www

permit tcp any any established

permit udp any eq domain any

permit icmp any any echo-reply

permit icmp any any unreachable

! "permit icmp any any time-exceeded" was rejected by Packet Tracer 9.0 (keyword not supported) and is NOT in the running config

deny ip any any

interface GigabitEthernet0/0

ip access-group EDGE-IN in

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 4

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## DSW-ACAD

! DSW-ACAD - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname DSW-ACAD

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

ip routing

ipv6 unicast-routing

ipv6 router ospf 10

router-id 10.10.254.2

exit

vlan 30

name ACAD-LAB

vlan 40

name STU-WIFI

vlan 50

name LIBRARY

vlan 70

name IOT

vlan 98

name MGMT-ACAD

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface Loopback0

description OSPF router-ID

ip address 10.10.254.2 255.255.255.255

interface GigabitEthernet1/0/1

description L1 to R-EDGE G0/1

no switchport

ip address 10.10.255.2 255.255.255.252

no shutdown

interface GigabitEthernet1/0/2

description L3 inter-block link

no switchport

ip address 10.10.255.9 255.255.255.252

ipv6 address 2001:DB8:ACAD:FF00::1/64

ipv6 ospf 10 area 0

no shutdown

interface range GigabitEthernet1/0/3 - 5

switchport mode trunk

switchport trunk native vlan 999

switchport nonegotiate

no shutdown

interface GigabitEthernet1/0/3

description Trunk to ASW-LAB Fa0/1

switchport trunk allowed vlan 30,98,999

interface GigabitEthernet1/0/4

description Trunk to ASW-LIB Fa0/1

switchport trunk allowed vlan 40,50,98,999

interface GigabitEthernet1/0/5

description Trunk to ASW-IOT Fa0/1

switchport trunk allowed vlan 70,98,999

interface range GigabitEthernet1/0/6 - 24

switchport mode access

switchport access vlan 998

shutdown

ip access-list extended STUDENT-IN

permit udp any any eq bootps

permit tcp any 10.10.3.0 0.0.0.127 established

permit icmp any 10.10.3.0 0.0.0.127 echo-reply

deny ip any 10.10.3.0 0.0.0.127

deny ip any 10.10.3.192 0.0.0.63

deny ip any 10.10.4.0 0.0.0.31

deny ip any 10.10.4.64 0.0.0.31

permit ip any any

ip access-list extended IOT-PROTECT

permit udp any any eq bootps

permit ip 10.10.4.0 0.0.0.31 host 10.10.4.36

permit icmp 10.10.4.0 0.0.0.31 10.10.4.64 0.0.0.31 echo-reply

deny ip any any

interface Vlan30

description Gateway ACAD-LAB

ip address 10.10.2.1 255.255.255.0

ip helper-address 10.10.4.34

ip access-group STUDENT-IN in

no shutdown

interface Vlan40

description Gateway STU-WIFI

ip address 10.10.0.1 255.255.254.0

ip helper-address 10.10.4.34

ip access-group STUDENT-IN in

no shutdown

interface Vlan50

description Gateway LIBRARY dual-stack

ip address 10.10.3.129 255.255.255.192

ip helper-address 10.10.4.34

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:50::1/64

ipv6 ospf 10 area 0

no shutdown

interface Vlan70

description Gateway IOT

ip address 10.10.4.1 255.255.255.224

ip helper-address 10.10.4.34

ip access-group IOT-PROTECT in

no shutdown

interface Vlan98

description Management ACAD block

ip address 10.10.4.65 255.255.255.240

no shutdown

router ospf 10

router-id 10.10.254.2

passive-interface default

no passive-interface GigabitEthernet1/0/1

no passive-interface GigabitEthernet1/0/2

network 10.10.0.0 0.0.255.255 area 0

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 4

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## DSW-ADMIN

! DSW-ADMIN - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname DSW-ADMIN

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

ip routing

ipv6 unicast-routing

ipv6 router ospf 10

router-id 10.10.254.3

exit

vlan 10

name ADMIN

vlan 20

name FACULTY

vlan 60

name SERVERS

vlan 99

name MGMT-ADMIN

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface Loopback0

description OSPF router-ID

ip address 10.10.254.3 255.255.255.255

interface GigabitEthernet1/0/1

description L2 to R-EDGE G0/2

no switchport

ip address 10.10.255.6 255.255.255.252

no shutdown

interface GigabitEthernet1/0/2

description L3 inter-block link

no switchport

ip address 10.10.255.10 255.255.255.252

ipv6 address 2001:DB8:ACAD:FF00::2/64

ipv6 ospf 10 area 0

no shutdown

interface range GigabitEthernet1/0/3 - 4

switchport mode trunk

switchport trunk native vlan 999

switchport nonegotiate

no shutdown

interface GigabitEthernet1/0/3

description Trunk to ASW-ADMIN Fa0/1

switchport trunk allowed vlan 10,20,99,999

interface GigabitEthernet1/0/4

description Trunk to ASW-DC Fa0/1

switchport trunk allowed vlan 60,99,999

interface range GigabitEthernet1/0/5 - 24

switchport mode access

switchport access vlan 998

shutdown

interface Vlan10

description Gateway ADMIN-REGISTRAR

ip address 10.10.3.193 255.255.255.192

ip helper-address 10.10.4.34

no shutdown

interface Vlan20

description Gateway FACULTY

ip address 10.10.3.1 255.255.255.128

ip helper-address 10.10.4.34

no shutdown

interface Vlan60

description Gateway SERVERS dual-stack

ip address 10.10.4.33 255.255.255.240

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:60::1/64

ipv6 ospf 10 area 0

no shutdown

interface Vlan99

description Management ADMIN block

ip address 10.10.4.81 255.255.255.240

no shutdown

router ospf 10

router-id 10.10.254.3

passive-interface default

no passive-interface GigabitEthernet1/0/1

no passive-interface GigabitEthernet1/0/2

network 10.10.0.0 0.0.255.255 area 0

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 4

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## ASW-LAB

! ASW-LAB - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname ASW-LAB

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

vlan 30

name ACAD-LAB

vlan 98

name MGMT-ACAD

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface FastEthernet0/1

description Trunk to DSW-ACAD G1/0/3

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 30,98,999

switchport nonegotiate

interface range FastEthernet0/2 - 3

description Computer lab PCs

switchport mode access

switchport access vlan 30

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

spanning-tree portfast

spanning-tree bpduguard enable

interface range FastEthernet0/4 - 24 , GigabitEthernet0/1 - 2

switchport mode access

switchport access vlan 998

shutdown

interface Vlan1

shutdown

interface Vlan98

ip address 10.10.4.66 255.255.255.240

no shutdown

ip default-gateway 10.10.4.65

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 15

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## ASW-LIB

! ASW-LIB - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname ASW-LIB

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

vlan 40

name STU-WIFI

vlan 50

name LIBRARY

vlan 98

name MGMT-ACAD

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface FastEthernet0/1

description Trunk to DSW-ACAD G1/0/4

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 40,50,98,999

switchport nonegotiate

interface FastEthernet0/2

description Library research PC

switchport mode access

switchport access vlan 50

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

spanning-tree portfast

spanning-tree bpduguard enable

interface FastEthernet0/3

description AP-STUDENT (SSID PU-STUDENT)

switchport mode access

switchport access vlan 40

spanning-tree portfast

interface range FastEthernet0/4 - 24 , GigabitEthernet0/1 - 2

switchport mode access

switchport access vlan 998

shutdown

interface Vlan1

shutdown

interface Vlan98

ip address 10.10.4.67 255.255.255.240

no shutdown

ip default-gateway 10.10.4.65

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 15

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## ASW-IOT

! ASW-IOT - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname ASW-IOT

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

vlan 70

name IOT

vlan 98

name MGMT-ACAD

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface FastEthernet0/1

description Trunk to DSW-ACAD G1/0/5

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 70,98,999

switchport nonegotiate

interface range FastEthernet0/2 - 5

description Smart campus IoT devices

switchport mode access

switchport access vlan 70

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

spanning-tree portfast

spanning-tree bpduguard enable

interface range FastEthernet0/6 - 24 , GigabitEthernet0/1 - 2

switchport mode access

switchport access vlan 998

shutdown

interface Vlan1

shutdown

interface Vlan98

ip address 10.10.4.68 255.255.255.240

no shutdown

ip default-gateway 10.10.4.65

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 15

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## ASW-ADMIN

! ASW-ADMIN - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname ASW-ADMIN

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

vlan 10

name ADMIN

vlan 20

name FACULTY

vlan 99

name MGMT-ADMIN

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface FastEthernet0/1

description Trunk to DSW-ADMIN G1/0/3

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,99,999

switchport nonegotiate

interface FastEthernet0/2

description Registrar office PC

switchport mode access

switchport access vlan 10

interface FastEthernet0/3

description Faculty PC

switchport mode access

switchport access vlan 20

interface range FastEthernet0/2 - 3

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

spanning-tree portfast

spanning-tree bpduguard enable

interface range FastEthernet0/4 - 24 , GigabitEthernet0/1 - 2

switchport mode access

switchport access vlan 998

shutdown

interface Vlan1

shutdown

interface Vlan99

ip address 10.10.4.82 255.255.255.240

no shutdown

ip default-gateway 10.10.4.81

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 15

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## ASW-DC

! ASW-DC - configuration script applied through the Packet Tracer IOS CLI (Paste) on 29 Oct 2026.

! This is the script that was entered, not a 'show running-config' export. Lab-only credentials: change before reuse.

!

enable

configure terminal

hostname ASW-DC

no ip domain-lookup

ip domain-name pu.internal

service password-encryption

enable secret PULab2026

username netadmin privilege 15 secret PULab2026

banner motd #Authorized access only - Premier University campus network (simulation)#

vlan 60

name SERVERS

vlan 99

name MGMT-ADMIN

vlan 998

name PARKING

vlan 999

name NATIVE

exit

interface FastEthernet0/1

description Trunk to DSW-ADMIN G1/0/4

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 60,99,999

switchport nonegotiate

interface range FastEthernet0/2 - 3 , FastEthernet0/5 - 6

description Data center servers

switchport mode access

switchport access vlan 60

spanning-tree portfast

spanning-tree bpduguard enable

interface FastEthernet0/4

description NMS-PC network management workstation

switchport mode access

switchport access vlan 99

spanning-tree portfast

spanning-tree bpduguard enable

interface range FastEthernet0/7 - 24 , GigabitEthernet0/1 - 2

switchport mode access

switchport access vlan 998

shutdown

interface Vlan1

shutdown

interface Vlan99

ip address 10.10.4.83 255.255.255.240

no shutdown

ip default-gateway 10.10.4.81

ip access-list standard MGMT-ONLY

permit 10.10.4.64 0.0.0.31

line con 0

logging synchronous

line vty 0 15

access-class MGMT-ONLY in

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

end

write memory

## End devices and services

End devices and services configured through the Packet Tracer GUI (Config / Services / Desktop tabs)

=====================================================================================================

SRV-DHCP-DNS  10.10.4.34/28  GW 10.10.4.33  DNS 10.10.4.34

DHCP (service ON). Pools: VLAN10-ADMIN gw 10.10.3.193 start 10.10.3.201 /26 max 54

VLAN20-FACULTY gw 10.10.3.1 start 10.10.3.11 /25 max 116

VLAN30-ACAD-LAB gw 10.10.2.1 start 10.10.2.11 /24 max 244

VLAN40-STU-WIFI gw 10.10.0.1 start 10.10.0.11 /23 max 500

VLAN50-LIBRARY gw 10.10.3.129 start 10.10.3.136 /26 max 55

VLAN70-IOT gw 10.10.4.1 start 10.10.4.11 /27 max 20

(default serverPool left in place; unused because no DHCP clients exist in VLAN 60)

DNS (service ON): portal.pu.internal A 10.10.4.35 | iot.pu.internal A 10.10.4.36 | nms.pu.internal A 10.10.4.37

www.internet.sim A 198.51.100.10 | portal6.pu.internal AAAA 2001:DB8:ACAD:60::35

SRV-WEB       10.10.4.35/28  GW 10.10.4.33 | 2001:DB8:ACAD:60::35/64 GW FE80::1 | HTTP index.html = campus portal page

SRV-IOT       10.10.4.36/28  GW 10.10.4.33 | IoT registration service ON | account iotadmin / PUIoT2026 (lab only)

Rules: R1-Lights-On  IF IOT-MOTION-1 On = true  THEN IOT-LIGHT-1 Status = On

R1-Lights-Off IF IOT-MOTION-1 On = false THEN IOT-LIGHT-1 Status = Off

R2-Fan-High   IF IOT-TEMP-1 Temperature > 30 C THEN IOT-FAN-1 Status = High

R2-Fan-Off    IF IOT-TEMP-1 Temperature < 26 C THEN IOT-FAN-1 Status = Off

SRV-NMS       10.10.4.37/28  GW 10.10.4.33 | Syslog service ON (no device has been pointed at it yet)

INTERNET-SRV  198.51.100.10/24 GW 198.51.100.1 | HTTP test page

NMS-PC        10.10.4.90/28  GW 10.10.4.81  DNS 10.10.4.34 (static)

PC-LAB1, PC-LAB2, PC-ADMIN1, PC-FAC1: DHCP (leases observed 10.10.2.11, 10.10.2.12, 10.10.3.201, 10.10.3.11)

PC-LIB1       DHCP IPv4 (10.10.3.136) + static IPv6 2001:DB8:ACAD:50::10/64, gateway FE80::1

AP-STUDENT    Port 1: SSID PU-STUDENT, WPA2-PSK, AES, passphrase PUStudent2026 (lab only)

STU-TABLET1, STU-PHONE1: SSID PU-STUDENT WPA2-PSK AES, DHCP (leases 10.10.0.12, 10.10.0.13)

IoT devices (IOT-MOTION-1, IOT-LIGHT-1, IOT-TEMP-1, IOT-FAN-1): adapter changed to PT-IOT-NM-1CFE (wired),

DHCP, IoT Server = Remote Server 10.10.4.36 (iotadmin). IOT-MOTION-1 leased 10.10.4.11.